AARYA V S (192512067)

1.B CHARACTERISTICS STUDY OF SIDE BAND POWER AND FREQUENCIES IN AM USING MATLAB

Objectives

PROGRAM (MATLAB Code)

clc;

clear; close all; Am=1; fm=500; Ac=5; fc=5000;

fs=20*fc; t=0:1/fs:4/fm;

m = Am*sin(2*pi*fm*t); c = Ac*sin(2*pi*fc*t); mu = Am/Ac;

am = Ac*(1+mu*sin(2*pi*fm*t)).*sin(2*pi*fc*t); dem = abs(hilbert(am)); dem = dem-mean(dem); N=length(am); f=linspace(-fs/2,fs/2,N); AM_FFT=abs(fftshift(fft(am)))/N;

USB=fc+fm; LSB=fc-fm; Pc=Ac^2/2; Psb=(mu^2*Pc)/4;

figure('Name','AM Modulation','NumberTitle','off');

subplot(3,2,1), plot(t,m), title('Message'), grid on

subplot(3,2,2), plot(t,c), title('Carrier'),

grid on subplot(3,2,3), plot(t,am), title('AM Signal'), grid on subplot(3,2,4), plot(t,dem), title('Demodulated'), grid on

subplot(3,2,5), plot(f,AM_FFT), xlim([fc-3*fm fc+3*fm]),

title('Sidebands'), grid on subplot(3,2,6), bar([Pc Psb Psb]), set(gca,'XTickLabel',{'Carrier','USB','LSB'}), title('Power'), grid on fprintf('μ=%.2f	|	USB=%d	Hz	|	LSB=%d	Hz	|	Pc=%.2f	W	|

Psb=%.3f W\n',mu,USB,LSB,Pc,Psb);

OUTPUT